# 🚛 Safety During Transit for Car Carrier Drivers

## 1. Pre-Trip Inspection (Critical for Auto Haulers)

Before departure, conduct a detailed inspection:

### 🔎 Tractor & Trailer

- Check brakes (tractor + trailer)

- Inspect tires (proper inflation, no cuts/bulges)

- Verify lights and reflectors

- Inspect hydraulic systems for deck movement

- Ensure ramps and locking pins are secured

Follow regulations under the Federal Motor Carrier Safety Administration if operating in the U.S.

### 🚗 Vehicle Load Inspection

- Confirm all vehicles are:

  - Properly positioned for weight distribution

  - Secured with straps, chains, or wheel nets

  - In park with parking brake engaged

- Check for fluid leaks (fuel, oil, coolant)

- Verify vehicle height clearance (bridges, tunnels)

## 2. Proper Load Securement

Car carriers are high-risk due to elevated decks.

- Use DOT-approved tie-downs.

- Recheck securement after first 50–100 miles.

- Distribute weight evenly to avoid:

  - Trailer sway

  - Axle overload

  - Rollover risk

- Ensure compliance with cargo securement rules in your region.

Poor load balance significantly increases rollover risk.

## 3. Height & Clearance Awareness (Major Risk Area)

Car carriers often exceed normal trailer height.

- Know your loaded height before departure.

- Check bridge and tunnel clearances.

- Avoid low overpasses.

- Watch tree branches and power lines in urban or rural areas.

Height miscalculations are one of the most common car-carrier incidents.

## 4. Driving Safety Practices

### ⚠️ Increased Rollover Risk

Because of a high center of gravity:

- Reduce speed on curves and ramps.

- Slow down before sharp turns.

- Avoid sudden steering inputs.

### 🚦 Defensive Driving

- Maintain longer stopping distance.

- Anticipate traffic changes.

- Avoid hard braking.

### 🌧️ Weather Adjustments

- Reduce speed in rain, snow, or high winds.

- Be cautious of crosswinds (top deck vehicles catch wind).

- Avoid icy inclines when fully loaded.

## 5. Loading & Unloading Safety

Many injuries occur during loading, not driving.

- Wear PPE (gloves, safety boots, reflective vest).

- Use 3-point contact when climbing ramps.

- Avoid standing under raised decks.

- Ensure hydraulic locks are engaged before working beneath.

## 6. Fatigue Management

Long hours increase accident risk.

- Follow Hours-of-Service rules.

- Take mandatory rest breaks.

- Avoid driving while drowsy.

- Stay hydrated and properly nourished.

## 7. Emergency Preparedness

- Carry fire extinguisher (accessible location).

- Keep wheel chocks available.

- Have reflective triangles/flares.

- Know spill response procedure for leaking vehicles.

- Keep emergency contacts readily available.

## 8. Theft & Security Prevention

Car carriers transport high-value cargo.

- Park in well-lit, secure areas.

- Lock vehicle keys in secure lockbox.

- Avoid posting real-time location on social media.

- Use GPS tracking when available.

## 9. Post-Trip Inspection

After delivery:

- Inspect for damage.

- Document condition with photos.

- Check straps and equipment wear.

- Report mechanical or hydraulic issues immediately.

# ⚠️ Key Risk Factors Unique to Car Carriers

- High center of gravity

- Load shifting

- Overhead clearance strikes

- Hydraulic deck failures

- Climbing/fall injuries during loading